Hackathon Challenge

Intelligent Pipeline Inspection Data Alignment & Corrosion Growth Prediction

# Executive Summary

Pipeline operators across the oil and gas industry face a critical challenge: effectively comparing inspection data collected at different points in time to understand how pipeline defects evolve. This hackathon challenges participants to build an automated solution for aligning In-Line Inspection (ILI) datasets and predicting corrosion growth—a problem that directly impacts pipeline safety, regulatory compliance, and operational efficiency.

# Industry Background

## What is In-Line Inspection?

In-Line Inspection (ILI) tools, commonly called "smart pigs," are sophisticated devices that travel through pipelines to detect and measure anomalies such as:

- Metal loss (corrosion, erosion)

- Dents and deformations

- Cracks and crack-like features

- Manufacturing defects

- Weld anomalies

These tools use technologies including magnetic flux leakage (MFL), ultrasonic testing (UT), and caliper measurements to create detailed maps of pipeline condition.

## The Scale of the Problem

- The U.S. alone has over 2.6 million miles of oil and gas transmission pipelines

- A single ILI run on a major transmission line can identify thousands to tens of thousands of anomalies

- Pipelines are typically inspected every 5-10 years (or more frequently based on risk)

- Each inspection generates massive datasets that must be compared to prior runs

## Why This Matters

| Impact Area | Consequence of Poor Data Alignment |
| --- | --- |
| Safety | Failure to identify rapidly growing defects could lead to leaks or ruptures |
| Regulatory | Inaccurate growth rates lead to non-compliance with integrity management requirements |
| Financial | Unnecessary excavations cost $50K-$500K+ each; missed defects cost far more |
| Operational | Manual alignment is time-consuming, delaying critical repair decisions |

# Problem Statement

## The Core Challenge

When a pipeline is inspected multiple times, each ILI run produces an independent dataset. These datasets are not automatically aligned—the same physical anomaly may appear at slightly different reported locations in each run due to:

- Odometer drift — The distance measurement wheel on the ILI tool can slip or accumulate error over long distances

- Tool speed variations — Different run conditions affect measurement consistency

- Reference point changes — Welds, valves, or other markers may be added, removed, or re-identified differently

- Tool technology differences — Different vendors or tool generations may characterize anomalies differently

- Anomaly evolution — Defects grow, merge, or new ones form between inspections

## Current State (Manual Process)

Today, skilled integrity engineers manually align datasets by:

- Identifying common reference points (girth welds, valves, tees)

- Creating "alignment zones" between fixed references

- Applying linear or non-linear stretch/compression to match segments

- Manually reviewing anomaly pairs based on multiple attributes

- Flagging unmatched anomalies for further review

This process can take weeks to months for a single pipeline segment and is prone to human error and inconsistency.

# Technical Requirements

## Input Data Elements

Participants should expect ILI data to include (at minimum):

| Data Field | Description | Example |
| --- | --- | --- |
| feature_id | Unique identifier for each anomaly | ML-00142 |
| distance | Absolute distance from pipeline start (feet or meters) | 15234.56 |
| odometer | Raw odometer reading from tool | 15198.23 |
| joint_number | Pipe joint (section) where anomaly exists | 847 |
| relative_position | Distance from upstream girth weld | 12.4 ft |
| clock_position | Circumferential position (o'clock) | 4:30 |
| feature_type | Classification of anomaly | External Metal Loss |
| depth_percent | Depth as percentage of wall thickness | 32% |
| length | Axial length of anomaly | 4.2 inches |
| width | Circumferential width | 2.1 inches |
| wall_thickness | Nominal or measured wall thickness | 0.375 inches |
| weld_type | Type of nearest weld | Girth Weld |

## Reference Data (Fixed Points)

| Element | Description |
| --- | --- |
| Girth welds | Circumferential welds joining pipe sections |
| Valves | Mainline valves at known locations |
| Fittings | Tees, elbows, reducers |
| Casings | Road/rail crossing casings |
| Above-ground markers | Survey stations with GPS coordinates |

# Solution Requirements

## Minimum Viable Product (MVP)

The hackathon solution should, at minimum:

- Ingest two ILI datasets from the same pipeline segment (provided in CSV/Excel format)

- Align reference points — Match girth welds and other fixed features between runs

- Apply distance correction — Adjust anomaly positions to a common reference frame

- Match anomalies — Pair anomalies from Run 1 to Run 2 based on corrected distance, clock position, feature type compatibility, and dimensional similarity

- Calculate growth rates — For matched pairs, compute depth growth rate, length growth rate, and width growth rate

- Flag exceptions — Identify new anomalies, missing anomalies, and uncertain matches

- Output results — Generate a matched dataset with growth calculations

## Stretch Goals

Participants are encouraged to extend their solutions with:

| Feature | Description |
| --- | --- |
| Confidence scoring | Assign probability/confidence to each match |
| Machine learning matching | Train models on historical matched data to improve accuracy |
| Growth rate prediction | Predict future corrosion rates using environmental or operational data |
| Anomaly clustering | Identify interaction zones where multiple anomalies may affect integrity |
| Visualization dashboard | Interactive tool to review alignments and matches |
| Multi-run analysis | Extend to 3+ ILI runs to establish growth trends |
| API integration | Build RESTful API for integration with integrity management systems |
| New anomaly prediction | Use ML to predict where new corrosion is likely to form |

# Evaluation Criteria

Solutions will be judged on:

| Criterion | Weight | Description |
| --- | --- | --- |
| Accuracy | 30% | Correct matching of anomalies (validated against expert-aligned data) |
| Completeness | 20% | Handles edge cases, exceptions, and data quality issues |
| Usability | 20% | Intuitive interface, clear outputs, easy to understand |
| Innovation | 15% | Creative approaches, novel algorithms, stretch goal implementation |
| Scalability | 10% | Performance on large datasets (10,000+ anomalies) |
| Documentation | 5% | Code quality, comments, and explanation of approach |

# Sample Data Description

Hackathon participants will receive:

- ILI Run 1 Dataset — Historical baseline inspection

- ILI Run 2 Dataset — Recent inspection (conducted 5-7 years after Run 1)

- Reference Alignment Points — Known fixed features with surveyed locations

- Expert-Matched Subset — A portion of manually matched anomalies for validation

- Data Dictionary — Detailed field descriptions and units

Note: Data will be synthetic or anonymized to protect operator confidentiality.

# Business Value

## Quantified Benefits

| Benefit | Estimated Value |
| --- | --- |
| Time savings | Reduce alignment time from weeks to hours (80-90% reduction) |
| Accuracy improvement | Reduce matching errors from ~10-15% to <5% |
| Dig avoidance | Prevent 5-10 unnecessary excavations per 100 miles (~$250K-$500K savings) |
| Risk reduction | Earlier identification of fast-growing defects prevents failures |
| Consistency | Standardized process reduces analyst-to-analyst variability |

## Strategic Value

- Predictive integrity management — Move from reactive to predictive maintenance

- Data-driven decisions — Quantified growth rates support risk-based prioritization

- Regulatory readiness — Automated documentation supports compliance audits

- Integration potential — Foundation for broader digital twin and asset management initiatives

# Technical Considerations

## Algorithmic Approaches to Explore

- Dynamic Time Warping (DTW) — Align sequences with non-linear distortion

- Iterative Closest Point (ICP) — Point cloud registration techniques

- Hungarian Algorithm — Optimal assignment for bipartite matching

- Graph-based matching — Model anomalies as nodes, relationships as edges

- Ensemble methods — Combine multiple matching criteria with weighted voting

- Deep learning — Siamese networks or transformers for similarity matching

## Data Quality Challenges

Participants should anticipate:

- Missing or null values in certain fields

- Inconsistent feature type naming conventions between vendors

- Clock position ambiguity (reported as decimal vs. time format)

- Unit inconsistencies (metric vs. imperial)

- Anomalies that split or merge between inspections

# Deliverables

Each team should submit:

- Working code — Functional solution with source code

- Documentation — README with setup instructions and approach description

- Demo — Live demonstration or recorded walkthrough

- Results file — Matched anomaly output on provided test data

- Presentation — 5-10 minute pitch covering approach, results, and potential extensions

# Resources

## Recommended Reading

- ASME B31.8S — Managing System Integrity of Gas Pipelines

- API 1163 — In-Line Inspection Systems Qualification

- NACE SP0502 — Pipeline External Corrosion Direct Assessment Methodology

- PHMSA Integrity Management regulations (49 CFR 192/195)

## Useful Libraries/Tools

- Python: pandas, scikit-learn, scipy, networkx, streamlit

- R: dplyr, caret, igraph, shiny

- Visualization: Plotly, D3.js, Tableau

- Machine Learning: TensorFlow, PyTorch, XGBoost

# Appendix A: Glossary

| Term | Definition |
| --- | --- |
| Anomaly | Any deviation from nominal pipe condition (defect, feature) |
| Clock position | Circumferential location expressed as hours on a clock face (12:00 = top of pipe) |
| Girth weld | Circumferential weld joining two pipe sections |
| ILI | In-Line Inspection |
| Metal loss | Reduction in wall thickness due to corrosion or erosion |
| Odometer | Distance measurement device on ILI tool |
| Smart pig | Colloquial term for ILI tool |
| Wall thickness | Thickness of pipe wall, typically measured in inches or mm |

# Appendix B: Sample Matching Logic (Pseudocode)

FOR each anomaly A in Run2:

candidates = []

FOR each anomaly B in Run1:

IF distance_difference(A, B) < distance_tolerance AND

clock_difference(A, B) < clock_tolerance AND

feature_types_compatible(A, B):

score = calculate_similarity(A, B)

candidates.append((B, score))

IF len(candidates) == 1:

match(A, candidates[0])

ELIF len(candidates) > 1:

best = select_best_match(candidates)

match(A, best) with uncertainty_flag

ELSE:

flag_as_new_anomaly(A)

FOR each unmatched anomaly in Run1:

flag_as_missing_anomaly()

Good luck, and remember: your solution could help prevent the next pipeline incident!